2
	[image: image1.jpg](FREATER
DANDENON( 5
City of Opportunity





	
SCHEDULE 17

Environmental Compliance (Planning) Guidelines



1.
Legal Compliance

The contract must be delivered in accordance with legislation, policies, regulations and Council’s Environmental Code for Tendering.

The contractor is to be provided with a copy of Council’s Environmental Management Policy and given the opportunity to peruse Council’s Legal Aspects Register.


The contract superintendent must have access to copies of all relevant policies, technical guidelines, codes of practice and other relevant documentation.

2.
Environmental Program

A completed Environmental Program is to be provided 

by contractors within 30 days of a Tender being awarded.

The Environmental Program must be relevant to activities 

carried out under the contract and potential incidents.

The Environmental Program shall: -

· Identify major activities to be performed under the contract;

· Identify activities that pose significant environmental risks;

· Document the environmental legislative responsibilities relating to tenderers activities;

· Identify the impacts likely to arise from any sub-contractors and consultants;

· Document how the environmental impacts shall be addressed, managed and minimised;

· List all permits and licences – including processes for illustrating how these permits and licences shall be adhered to and monitored;

· Copies of all licences and permits are to be attached to the Environmental Program; and

· Indicate how environment aspects are covered in Emergency procedures that shall be documented under requirements for Occupational Health and Safety.

The Environmental Program is to be forwarded to Council officer responsible for Environmental Issues within 2 working days for comment and approval.

3.
Checklist

The contract superintendent is to develop a checklist that covers matters noted in the Environmental Program. This checklist is to be used for site audits and monthly reporting.

4.
Site Audits/Visits

During site visits the following issues are to be inspected and reported:

Spill equipment
Adequate and appropriate spill equipment is to be accessible to cover the largest possible spill or leak.

Spills

Check that there is no evidence of spills or leaks.  

Bunding
Temporary or permanent bunding is to be provided to ensure that no pollutants (including sediment, chemicals, oils, cement, bitumen and dirty water) can enter the stormwater drainage system.  Temporary bunding includes hay bales, sand bags, commercially produced “sausages” or “pillows” or mounds of sand or soil.

Liquid run-off
All liquid waste must be disposed of to landfill or to sewer if a trade waste agreement has been negotiated with the relevant water authority.

Only clean water is to enter the stormwater drainage system.

Solid waste
Solid waste must be stored so that it cannot cause litter or pollution of land or water.  Containers must be provided and used. 

Dust control
Dust from works must be controlled by using a light spray of water.  [The use of oil or oil based substances to suppress dust is illegal]. 

Noise
Check that the contractors are abiding by noise limits and operational times as set down in their contract.  All vehicles and equipment should meet occupational and health standards and be fitted with approved mufflers.

Burning off
No waste is to be burnt unless a written permit has been issued by Residential Amenity.

Flora and Fauna
Are the works disturbing, or likely to disturb significant plants or the habitat of native animals or birds?  Check that the contractor has liaised with Council officers regarding the potential impact on flora and fauna.

5.
Monthly Reports 


The following must be provided in addition to the checklist:

5.1
Copies of all landfill and waste disposal dockets.  Dockets must be checked to ensure the type and amount of waste is appropriate for the works undertaken under the contract. [Insufficient documentation may indicate illegal disposal]


Prescribed waste such as oil, tar, bitumen, filter backwash, sludges, triple interceptor wastes and most chemicals must be transported and disposed of to a secure licensed landfill or waste treatment company approved by the Environment Protection Authority.

5.2
A detailed report of any incidents such as spills or leaks.  Reports must include the type and quantity of the spill or leak and actions taken to remedy the situation.

5.3
Trade Waste Agreements.  A written report must be provided to demonstrate adherence with the conditions of any Trade Waste Agreement or Greasy Waste Agreement.

5.4
EPA Permits and licences.  A written report must be provided to demonstrate adherence with the conditions of any EPA license or permit.

5.5
Complaints.  A report detailing any complaints about the environmental performance of the contractors and how these issues have been addressed
Refer to Council’s

Environmental Code for Tendering 1998

for further details.

For further information Contact

Council Officer responsible for environmental issues

[image: image1.jpg]
Distribution List:


Superintendent Rep to save in Objective\Contract Management File








Page 3 of 3

